Water & Drainage - Frequently Asked Questions during Bad Weather Conditions

Frozen Pipes

Householders experiencing problems with frozen pipes are advised to contact their plumber.

If you have no water and you have confirmed with the Council that there is no problem with the mains supply in your area, it is likely that you have frozen pipes on your property or in your home. 

It is critical to know where your stopcock is located in your property. If a leak becomes apparent then shut off the stopcock or alternative isolating valve that may be in your house to minimise potential damage.

If there is no water from the cold tap in your kitchen, the chances are that the stopcock or the line from there to the house is frozen. It may help to pour warm water over the stopcock - do not use boiling water for this purpose or any naked flame. Remember, however, that if water at any temperature builds up in the stopcock chamber it will almost certainly freeze within minutes: so use as little water as possible. If the freezing has occurred in the line between the stopcock and the house, the warm water is unlikely to solve the problem. 

If you can identify the pipe that is frozen and it is above ground you may carry out the following to try to thaw it out;

• You may try to gently heat the pipe where you suspect the freeze has occurred. Alternatively, you could wrap a towel around the pipe securing it with duct tape and pour hot water over the towel. 

• Once the obstruction is cleared and the water is flowing again, turn off all taps and check for leaks in the area where the obstruction occurred. Any leaks you find should be repaired immediately. If you cannot repair them yourself, call a qualified plumber.

Remember, if you have a burst pipe or frozen supply, to be careful about the use of heating systems, washing machines and other water-dependent appliances and facilities. If you have a problem or concern, contact a qualified plumber for advice.

- How can you help prevent the problem of frozen pipes?
To prevent problems, do what you can to protect the pipes inside and outside your house. 

• Where possible ensure any outside pipes are insulated and if not insulated at least covered. 

• For houses with their storage tanks in the attic, you should ensure that the area under the roof tank can get heat from underneath, so remove any insulation which prevents that. It may be useful to open the trap door to the attic to allow heat up there (only if safe to do so) 

• It can also be useful to allow heat to circulate to under-sink pipes where they run along an outside wall by leaving a cupboard door open (only if safe to do so).

My water supply was off recently and the water is brown coming from tap? 
When the water supply has been switched off to carry out repairs to bursts etc., some sediment can become dislodged from the water pipes during the switching back on of the supply.  If you run the water and its brown please do not run the water again for 30mins giving the sediment time to settle. You can continue this process for a couple of hours if the water is still brown please contact SDCC.  

My water supply was off recently and the water appears cloudy coming from tap? 
This is quite a common occurrence when a water main has been opened to repair bursts etc., air enters the pipe network and gives the water a milky/cloudy appearance when it comes from the tap.  If you leave the water to settle in glass you will see that the air quickly leaves and the water is as clear as normal. 

My water is off at the moment, can I use my central heating system? 
If your central heating system has been installed by a registered contractor it will be run from a separate expansion tank in the attic.  This water will not be affected by a water main shutdown and it is safe to run the system. 

My water was off recently and although water has come back to the cold tap in the kitchen there is no water upstairs in the bathroom? 
This occurs in some plumbing systems after a water shutdown.  It is an airlock in your system and you may need to engage a plumber to eliminate it.   SDCC endeavour to keep shutdown of supplies to a minimum but do not maintain interior plumbing systems

Emergency Information & updates or reporting a Complaint:

South Dublin County Council Head Office

Daytime 01-4149274 or 01-4149000 between

09:00 -17:00 Monday – Thursday

09:00-16.30 Friday

Deansrath Depot - 01- 413 8500 between (08:00-24:00)

Emergency Out of Hours 01-2910 975 (00:00 – 08:00)

ONLINE:

Fix your Street: http://www.fixyourstreet.ie/main

Customer Care Online: http://www.sdublincoco.ie/index.aspx?pageid=1279&proceed=true